SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1-CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1-Constraint-Based Problem Solving |
| Weightage: | 60% | Total Marks: | 25 |
| CO5Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | C.ABHINAY ADITHYA | Reg.No.: | 192572115 |
| Department: | CSE WITH AI | Date: |  |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

Given Constraints

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select the best I/O method for minimum latency

Solution

For this real-time embedded system, use Memory-Mapped I/O with Vectored Interrupts and DMA/Buffering.

- Memory-Mapped I/O is selected because sensor registers can be accessed directly using normal memory instructions. This provides lower latency and faster access than I/O-mapped I/O.

- Vectored Interrupt Handling is used by assigning each sensor a unique interrupt vector. When a sensor generates an interrupt, the CPU directly executes the corresponding ISR without checking all devices.

- Since sensors generate interrupts every 1 ms, the ISR must be very short. It should only:

  - Read the sensor data

  - Store it in a buffer

  - Clear the interrupt

  - Return immediately

- For large or high-speed data transfers, DMA should be used. DMA transfers data directly between the device and memory, reducing CPU involvement.

- Heavy data processing is performed later by a background task, ensuring that interrupt handling consumes less than 15% CPU utilization.

Final Design

Sensors → Interrupt Controller → Vector Table → ISR → Buffer/DMA → Memory → Background Processing

Conclusion

Thus, Memory-Mapped I/O + Vectored Interrupts + Short ISR + DMA is the best solution. It ensures low latency, fast response to 1 ms interrupts, and CPU utilization below 15%, making it suitable for a real-time embedded system.

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

Problem Solution

To satisfy the given requirements, the system should use an NVMe SSD connected through PCIe, with DMA for bulk data transfer and interrupts for completion notification.

Architecture

CPU → NVMe Driver → PCIe/NVMe Controller → DMA → Main Memory

Working

- The CPU initializes an NVMe read/write command and places it in the NVMe submission queue.

- The command contains the memory addresses and transfer details.

- The NVMe controller uses DMA to transfer bulk data directly between:

  - NVMe SSD ↔ Main Memory

- The CPU does not copy individual data blocks, reducing CPU overhead.

- PCIe provides the required high-speed interface, enabling throughput of more than 3 GB/s.

- After the DMA transfer is complete, the NVMe controller sends an MSI/MSI-X interrupt to the CPU.

- The CPU's interrupt handler processes the completion queue and confirms that the I/O operation has finished.

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

Working of the Architecture

1. USB Communication

A USB Host Controller manages all 6 peripherals. Each device communicates through USB ports or a USB hub.

2. Thunderbolt Communication

Two high-speed devices are connected through the Thunderbolt controller, providing high bandwidth and low latency.

3. Hardware Interrupts

Hardware interrupts are generated when:

- A USB device completes an operation

- A Thunderbolt device completes a high-speed transfer

- An error occurs

The interrupt controller receives and manages these interrupts.

4. Software Interrupts

Software interrupts are used for:

- Device driver processing

- Deferred interrupt handling

- Scheduling lower-priority tasks

This prevents the CPU from spending too much time inside hardware interrupt handlers.

Avoiding Interrupt Starvation

Use a combination of:

- Priority-based interrupt handling

- Round-robin scheduling among devices of the same priority

- Interrupt aging

Interrupt Aging

If a low-priority interrupt waits for a long time, its priority is gradually increased. This ensures that no device waits indefinitely.

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

Problem

Design a data acquisition system that handles a continuous data stream of 500 MB/s with minimum CPU involvement. Compare DMA and interrupt-driven I/O, and use Memory-Mapped I/O (MMIO) for device registers.

Solution

Step 1: Select Memory-Mapped I/O

The device registers are mapped into the CPU's memory address space.

The CPU accesses:

- Control Register

- Status Register

- DMA Address Register

- Transfer Size Register

This allows fast configuration of the data acquisition device using normal memory operations.

Step 2: Analyze the Data Rate

The system receives:

Data Rate = 500 MB/s

Since the data stream is continuous, transferring every byte through the CPU would create excessive CPU load.

Therefore, Programmed I/O and frequent interrupt-driven data transfer are unsuitable.

Step 3: Use DMA for Bulk Data Transfer

The CPU initializes the DMA controller only once.

CPU → Configure MMIO Registers → Start DMA

Then:

Data Acquisition Device → DMA → Main Memory

The DMA controller transfers data directly into RAM without requiring the CPU to handle every byte.

Step 4: Use a Circular Buffer

A ring/circular buffer is created in main memory.

Buffer 1 → Buffer 2 → Buffer 3 → Buffer 4

↑                                  ↓

←──────────────────────────────────

DMA continuously fills the buffers, allowing continuous acquisition at 500 MB/s.

Step 5: Generate Interrupt After Buffer Completion

Instead of generating an interrupt for every data item:

- DMA fills one complete buffer.

- DMA generates one interrupt.

- CPU processes or schedules the completed buffer.

- DMA continues filling the next buffer.

This greatly reduces interrupt overhead.

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

A priority-based interrupt handling mechanism is used to support nested interrupts and achieve interrupt latency below 10 µs.

- High-priority devices use vectored interrupts, where the CPU directly jumps to the corresponding Interrupt Service Routine (ISR). This reduces interrupt identification time and helps achieve low latency.

- Nested interrupts are supported by allowing a higher-priority interrupt to interrupt a currently executing lower-priority ISR. The CPU saves the current context, executes the higher-priority ISR, and then restores the previous ISR.

- If an interrupt has lower priority than the currently executing ISR, it is kept in a pending queue until the higher-priority ISR completes.

- Software interrupts are used for background tasks such as logging, data processing, maintenance, and status updates. These tasks are deferred so they do not affect critical hardware interrupts.

- The ISR should be kept short to ensure that the maximum interrupt latency remains below 10 µs.

Final Solution

The system uses priority-based nested interrupts, vectored interrupts for high-priority devices, and software interrupts for background tasks. Higher-priority interrupts can preempt lower-priority ISRs, ensuring fast response and efficient interrupt handling while maintaining the required latency of less than 10 µs.

Code of Conduct:

I _C.ABHINAY ADITHYA_(192572115) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding(25%) | Constraint Analysis(25%) | Solution Development(25%) | Application of Concepts(15%) | Justification(10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |