SIMATS ENGINEERING

Department of Computer Science & Engineering ASSESSMENT TOOL-1 EXPERIMENTS

| Course Code: | CSA1221 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: | C.ABHINAY ADITHYA | Reg.No.: | 192572115 |
| Department: | CSE WITH AI | Date: |  |

ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic  arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple  registers to demonstrate how data is transferred between them during execution, and analyze the  sequence of operations and the role of the ALU in processing data.

A register transfer system is used to transfer data between multiple registers and perform arithmetic and logical operations using the ALU.

1. Registers Used

R1 – stores first input data.

R2 – stores second input data.

R3 – stores intermediate results.

R4 – stores final results.

2. Register Transfer Operations

Data can be transferred between registers using control signals.

R3 ← R1

R4 ← R2

R1 ← R2

Here, the arrow ← represents the transfer of data.

3. Arithmetic Operations

The ALU (Arithmetic Logic Unit) performs arithmetic operations.

Addition: R3 ← R1 + R2

Subtraction: R3 ← R1 − R2

For example, if R1 = 10 and R2 = 5:

- Addition → R3 = 15

- Subtraction → R3 = 5

4. Logic Operations

The ALU also performs logical operations.

- AND: R3 ← R1 AND R2

- OR: R3 ← R1 OR R2

These operations are performed bit-by-bit on the data stored in the registers.

5. Role of ALU and Execution Sequence

The execution sequence is:

Input Registers → ALU → Result Register

The control unit selects the required operation and controls data transfer between registers. The ALU receives data from R1 and R2, performs addition, subtraction, AND, or OR, and stores the result in R3/R4.

Conclusion: This system demonstrates how registers transfer data and how the ALU processes that data to perform arithmetic and logical operations.

- Create a model of a computer system using single-bus and multi-bus organization, and perform  data transfer operations between registers, memory, and I/O. Compare the time taken and  efficiency of both approaches, and analyze how multiple bus structures improve parallel data  transfer and reduce bottlenecks.

A bus organization defines how data is transferred between the CPU registers, memory, ALU, and I/O devices. In a single-bus organization, all registers, memory, and I/O devices share one common bus. Only one data transfer can take place through the bus at a time. For example, to transfer data from register R1 to R2, the data from R1 is placed on the bus and then loaded into R2. Similarly, memory and I/O data must use the same bus, which can create a bottleneck when many operations are required.

In a multi-bus organization, two or more buses are used to transfer data simultaneously. For example, one bus can transfer data from R1 while another bus transfers data from R2, and the ALU can perform an operation on both values at the same time. This allows parallel data transfer and reduces the number of clock cycles required for an operation.

| Feature | Single-Bus | Multi-Bus |
| --- | --- | --- |
| Number of buses | One | Two or more |
| Data transfer | Mostly sequential | Can be parallel |
| Speed | Lower | Higher |
| Hardware cost | Lower | Higher |
| Bottleneck | More | Less |
| Efficiency | Moderate | High |

For example, in a single-bus system, an operation such as R3 = R1 + R2 may require multiple clock cycles because R1 and R2 must use the same bus sequentially. In a multi-bus system, R1 and R2 can be transferred to the ALU through separate buses simultaneously, allowing the addition to be completed faster.

Thus, single-bus organization is simpler and cheaper, but it suffers from bus contention and slower data transfers. Multi-bus organization provides faster execution, better parallelism, and reduced bottlenecks, making it more efficient for high-performance computer systems.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and  execute a set of instructions. Measure the performance in terms of speedup and throughput, and  compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

A 5-stage instruction pipeline divides instruction execution into five stages: Fetch (IF), Decode (ID), Execute (EX), Memory (MEM), and Write-back (WB). In the Fetch stage, the instruction is fetched from memory. In Decode, the instruction is decoded and the required registers are identified. In Execute, the ALU performs the required operation. In the Memory stage, data is read from or written to memory when required. Finally, in the Write-back stage, the result is stored in the destination register.

For example, consider five instructions I1, I2, I3, I4, and I5. In a non-pipelined processor, each instruction must complete all five stages before the next instruction begins. Therefore, 5 instructions require 25 clock cycles. In a pipelined processor, different instructions can occupy different stages simultaneously.

| Clock Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I1 | IF | ID | EX | MEM | WB |  |  |  |  |
| I2 |  | IF | ID | EX | MEM | WB |  |  |  |
| I3 |  |  | IF | ID | EX | MEM | WB |  |  |
| I4 |  |  |  | IF | ID | EX | MEM | WB |  |
| I5 |  |  |  |  | IF | ID | EX | MEM | WB |

- For 5 instructions, the pipelined system completes execution in 9 clock cycles, while the non-pipelined system requires 25 clock cycles. The speedup is:

- Speedup = Non-pipelined time / Pipelined time = 25 / 9 ≈ 2.78

- The throughput is also improved because after the pipeline is filled, approximately one instruction can be completed every clock cycle. Thus, pipelining increases instruction throughput and makes better use of CPU resources.

- Conclusion: A 5-stage pipeline significantly improves processor performance by allowing multiple instructions to be processed simultaneously. Although pipeline hazards such as data, control, and structural hazards may introduce delays, pipelining generally provides higher speedup, throughput, and CPU efficiency than a non-pipelined system.

4. Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards  occur. Observe their impact on instruction execution, and implement techniques such as stalling,  data forwarding, and branch prediction to resolve these hazards and improve performance.

A pipeline hazard occurs when the next instruction cannot execute in the expected clock cycle. In a 5-stage pipeline consisting of Fetch (IF), Decode (ID), Execute (EX), Memory (MEM), and Write-back (WB), three major hazards can occur: data hazards, control hazards, and structural hazards.

Data Hazard: A data hazard occurs when one instruction depends on the result of a previous instruction. For example, I1: R1 = R2 + R3 followed by I2: R4 = R1 + R5. Here, I2 needs the value of R1 produced by I1. This can be resolved using data forwarding, where the result of I1 is directly forwarded to the execution stage of I2 without waiting for write-back. If forwarding is not possible, a stall or pipeline bubble is inserted.

Control Hazard: A control hazard occurs due to branch instructions. For example, I1: BEQ R1, R2, LABEL. The processor may not know which instruction to fetch next until the branch decision is made. Branch prediction can predict whether the branch is taken or not taken and continue fetching instructions. If the prediction is wrong, the incorrectly fetched instructions are removed from the pipeline.

Structural Hazard: A structural hazard occurs when two instructions require the same hardware resource at the same time. For example, if instruction fetch and data memory access require the same memory unit simultaneously, a conflict occurs. This can be resolved by stalling the pipeline or providing separate instruction and data memories.

| Hazard | Example | Resolution |
| --- | --- | --- |
| Data Hazard | I2 depends on I1 result | Data forwarding / Stall |
| Control Hazard | Branch instruction | Branch prediction |
| Structural Hazard | Same resource needed simultaneously | Stall / Additional hardware |

Conclusion: Pipeline hazards reduce performance by causing delays and reducing throughput. Stalling prevents incorrect execution, data forwarding reduces data-related delays, and branch prediction reduces control-hazard penalties. Proper use of these techniques improves pipeline efficiency and execution speed.

5. Analyze a processor supporting superscalar execution, including out-of-order and speculative  execution, and extend the study to include hyper-threading and simultaneous multithreading  (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance  improvements in terms of instruction throughput and resource utilization.

A superscalar processor can execute more than one instruction in a single clock cycle by using multiple execution units such as ALUs, floating-point units, and load/store units. For example, a 4-wide superscalar processor can potentially issue four independent instructions simultaneously, increasing instruction throughput compared with a single-issue processor.

Out-of-order (OoO) execution allows the processor to execute instructions when their required operands and resources are available rather than strictly following program order. For example, if Instruction 2 is waiting for data from Instruction 1, an independent Instruction 3 can execute first. This improves utilization of processor resources. Speculative execution allows the processor to predict the future execution path, especially branch outcomes, and execute instructions before the branch is confirmed. Incorrectly executed instructions are discarded.

Hyper-Threading/Simultaneous Multithreading (SMT) allows a single physical processor core to maintain and execute instructions from multiple threads. When one thread is waiting for memory or another resource, instructions from another thread can use otherwise idle execution units. Thus, SMT improves overall resource utilization and system throughput.

| Technique | Main Advantage | Effect |
| --- | --- | --- |
| Superscalar | Executes multiple instructions per cycle | Higher throughput |
| Out-of-order | Executes ready instructions first | Better resource utilization |
| Speculative execution | Executes predicted instructions | Reduces branch delays |
| SMT/Hyper-Threading | Executes multiple threads on one core | Better core utilization |

For example, if a processor executes 1 instruction per cycle without these techniques, its throughput is approximately 1 instruction/cycle. A 4-wide superscalar processor can theoretically achieve up to 4 instructions/cycle when enough independent instructions are available. With OoO and SMT, unused execution resources can be utilized more effectively, although the actual performance improvement depends on dependencies, memory delays, branch accuracy, and available parallelism.

Conclusion: Superscalar execution increases instruction-level parallelism, while out-of-order and speculative execution reduce delays and improve execution-unit utilization. SMT/Hyper-Threading further improves throughput by allowing multiple threads to share the resources of a processor core. Together, these techniques provide significantly better performance than a simple single-issue processor.

Code of Conduct:

I ____C.ABHINAY ADITHYA_ (Name / Reg No) certify that this submission is  my original work and that I have adhered to the guidelines specified for this assessment.I understand  that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Concepts(20%) | Experimental Design(20%) | Use of Tools(25%) | Analysis of Results(20%) | Documentation(15 %) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |